10-6-08 Hemodialysis

Dialysis Goals

- Filtration – replace filtration function of kidney, remove “uremic toxins”  use as markers

- Electrolyte Balance – maintain proper levels of  Na+, K+, HCO3-, Ca++, PO4

- Hormones – hormonal replacement  erythropoietin, vitamin D analogs, PTH control

ESRD Lifestyle Adjustments – Diet, Work/Travel, Comorbidities, Healthcare Support

- Diet – have large dietrary/fluid restrictions

- Travel & Work – limited by constraints of dialysis treatment

- Comorbidities – ESRD usually occurs in the context of other health problems such as DM complications

- Healthcare Support – multi-disciplinary team needed  nurses, physicians, technicians, dieticians, social

Dialysis Physiology – Diffusion/Convection (K/Na), Clearance/Ultrafiltration

- Diffusion/Convection – two different principles used in dialysis:

  - Diffusion – decreasing the concentration of substance across membrane  clearance

  - Convection – bulk flow of a fluid in/out of body  fluid removal

  - Na+ removed in saline through convection – pressure gradient between blood/dialysate

  - K+ removed by concentration gradients thru diffusion – conc. gradient between blood/dialysate

- Clearance/Ultrafiltration – two different objectives a dialysis machine can accomplish:

  - Fluid Removal – may need to remove 1L of fluid in order to stop pulmonary edema

  - Clearance – removing a solute, not solvent; e.g. clearing “30L” of urea ≠ removing 30L fluid

- Dialysate composition – electrolytes, glucose, heparin

Dialysis Process - Blood In, Diffusion/Convection

- Blood pumped into dialyzer, goes thru lots of tiny semi-permeable tubes in parallel

- Outside of the tubes, a dialysate is flowing through, removing urea/toxins diffusing out into here

- Saline (Na & H2O) moved by pressure gradient existing between the tubing & dialysate

Dialysis Physiology – Blood Pressure vs. Fluid Status, Vascular Access

- Blood Pressure vs. Fluid Status – need to keep blood pressure up while dialyzing a certain volume

  - Hypovolemia – too much fluid removed/fluid removed faster than equilibration  hypotension

  - Hypervolemia – if not on dialysis for too long, consume too much  hypertension

- Vascular Access – 3 types include double lumen catheter, prosthetic/native vein fistualae:

  - Central venous catheter:		Pros: immediate use	           Cons: thrombosis, infection

  - Prosthetic arteriovenous fistula:	Pros: less thrombosis/inf.	           Cons: less immediate use

  - Native arteriovenous fistula:	Pros: best longevity, no thromb    Cons: require maturation

- Drug dosing considerations – although 5 liters of circulation, much more in ECF/ICF  equilibration…

  - KT/V, URR (Urea reduction ratio)  terms relating to solute removal kinetics

  - Drug removal – need to know how fast drugs are removed by dialysis vs. kidney  dose adjust

  - Similar Drugs – do not always have similar removal rates (e.g. amoxicillin vs. nafcilin)

___________________________________________________________________________________________________________

Peritoneal Dialysis

Peritoneal Dialysis

- Peritoneal Dialysis – filter fluid through peritoneum

- Outpatient – peritoneal dialysis done at home, whereas hemodialysis usually done at a hospital/clinic

Peritoneal Dialysis Principles

- Peritoneal Membrane – used as filtration surface, SA equivalent to body SA (1.2 sq. m.), but…

  - Effective SA – depends on vascularity as well as actual SA  clearance ~ effective SA

- Solute Movement – driven by diffusion  peritoneal capillary blood to peritoneal dialysate

  - Concentration gradient, SA, Intrinsic Resistance – all affect diffusion in predictable ways

  - Molecular weight – solute MW also affects diffusion, smaller = faster

  - Removal and Supplement – can add high concentrations of chemicals in dialysate to diffuse in

- Fluid Movement – moves by convection, as controlled by fluid removal/addition

  - Ultrafiltration – dialysate is hyperosmolar (due to glucose), net effect is fluid movement out

  - Fluid Movement – rate given by glucose concentration, dwell time, & membrane resistance

    - Low resistance 	 bad at fluid movement, since [glucose] gradient dissipates!

 good at solute removal though, since diffusion easier

    - High resistance 	 good at fluid movment, since [glucose] gradient maintained!

 bad at solute removal though, since diffusion restricted

    - Glucose concentration – increase to remove more water

  - Hyperglycemia/extra calories – need to adjust insulin/diet to account for glucose diffusion

Peritoneal Dialysis Components/Techniques

- Catheter – port anchored into abdominal wall, flow into peritoneal cavity

- Dialysate – plastic bags of electrolytes, varying glucose concentration, no K+/urea/creatinine

- Continuous Ambulatory Peritoneal Dialysis (CAPD) – cycle during day, fill at night and empty in AM

- Automated Peritoneal Dialysis – fill in morning, work all day, come home, empty, overnight automated cycling

- Limitations – peritoneal fluid has high glucose, low pH  biocompatibility issues; also protein/AA loss

- Icodextrin – glucose polymer, still acts as osmotic agent, doesn’t diffuse  slow but sustained UF

- Complications – exit site infection, peritonitis, hernia development, peritoneal sclerosis, “burn out”

  - Peritonitis – 25% of hospitalizations in PD patients, 33% reason to transfer to hemodialysis